DAILY ASSESSMENT FORMAT

| Date: | 29-05-2020 | Name: | Akshata Madiwalar |
| --- | --- | --- | --- |
| Course: | Logic Design | USN: | 4AL17EC046 |
| Topic: | Analysis of Clocked Sequential circuits,Digital Clock Design | Semester & Section: | 6th sem & A sec |
| Github Repository: | Akshata-course |  |  |

FORENOON SESSION DETAILS

Report – Report can be typed or hand written for up to two pages Analysis of Clocked Sequential Circuits ● Some flip-flops have asynchronous inputs that are used to force the flip-flop to a particularstate independently of the clock ● The input that sets the flip-flop to 1 is called preset or direct set. The input that clears the flip-flop to 0 is called clear or direct reset ● The information available in a state table can be represented graphically in the form of a state diagram. In this type of diagram a state is represented by a circle and the (clock-triggered) transitions between states are indicated by directed lines connecting the circles. ● The time sequence of inputs, outputs, and flip-flop states can be enumerated in a state table (transition table). The table has four parts present state, next state, inputs and outputs. ● In general a sequential circuit with 'm' flip-flops and 'n' inputs needs 2m+n rows in the state table. Positive Edge Triggered D Flip-flop ● When the reset input is 0 it forces output Q' to Stay at 1 which clears output Q to 0 thus resetting the flip-flop. ● Two other connections from the reset input ensure that the S input of the third SR latch stays at logic 1 while the reset input is at 0 regardless of the values of D and Clk. ● Function table suggests that: ❏ When R = 0, the output is set to 0 (independent of D and Clk). ❏ The clock at Clk is shown with an upward arrow to indicate that the flip-flop triggers on the positive edge of the clock. ❏ The value in D is transferred to Q with every positive-edge clock signal provided that R = 1. Analysis with D Flip-Flops ● The input equation of a D Flip-flop is given by DA = A ⊕ x ⊕ y. DA means a D Flip-flop with output A. ● The x and y variables are the inputs to the circuit. No output equations are given, which implies that the output comes from the output of the flip-flop. ● The state table has one column for the present state of flip-flop 'A' two columns for the two inputs, and one column for the next state of A. ● The next-state values are obtained from the state equation A(t + 1) = A ⊕ x ⊕ y. ● The expression specifies an odd function and is equal to 1 when only one variable is 1 or when all three variables are 1. Analysis with JK Flip-Flops ● The circuit can be specified by the flip-flop input equations: ❏ JA = B; KA = Bx' ❏ JB = x'; KB = A'x + Ax' = A ⊕ x ● The next state of each flip-flop is evaluated from the corresponding J and K inputs and the characteristic table of the JK flip-flop listed as: ❏ When J = 1 and K = 0 the next state is 1 ❏ When J = 0 and K = 1 the next state is 0 ❏ When J = 0 and K = 0 there is no change of state and the next-state value is the same as that of the present state. ❏ When J = K = 1, the next-state bit is the complement of the present-state bit State Reduction and Assignment ● Two sequential circuits may exhibit the same input-output behavior but have a different number of internal states in their state diagram. ● Certain properties of sequential circuits may simplify a design by reducing the number of gates and flip-flops it uses. Reducing the number of flip-flops reduces the cost of a circuit. ● The reduction in the number of flip-flops in a sequential circuit is referred to as the state reduction problem. State-reduction algorithms are concerned with procedures for reducing the number of states in a state table while keeping the external input-output requirements unchanged

.

| Date: | 29-05-2020 | Name: | Akshata Madiwalar |
| --- | --- | --- | --- |
| Course: | Python | USN: | 4AL17EC046 |
| Topic: | Object Oriented programming | Semester & Section: | 6th sem & A sec |
| AFTERNOON SESSION DETAILS |  |  |  |
| Report – Report can be typed or hand written for up to two pages. Here are the frontend.py and backend.py scripts in OOP style. To execute this program you should execute the frontend.py file. #frontend.py from tkinter import * from backend import Database database=Database("books.db") class Window(object): def __init__(self,window): self.window = window self.window.wm_title("BookStore") l1=Label(window,text="Title") l1.grid(row=0,column=0) l2=Label(window,text="Author") l2.grid(row=0,column=2) l3=Label(window,text="Year") l3.grid(row=1,column=0) l4=Label(window,text="ISBN") l4.grid(row=1,column=2) self.title_text=StringVar() self.e1=Entry(window,textvariable=self.title_text) self.e1.grid(row=0,column=1) self.author_text=StringVar() self.e2=Entry(window,textvariable=self.author_text) self.e2.grid(row=0,column=3) self.year_text=StringVar() self.e3=Entry(window,textvariable=self.year_text) self.e3.grid(row=1,column=1) self.isbn_text=StringVar() self.e4=Entry(window,textvariable=self.isbn_text) self.e4.grid(row=1,column=3) self.list1=Listbox(window, height=6,width=35) self.list1.grid(row=2,column=0,rowspan=6,columnspan=2) sb1=Scrollbar(window) sb1.grid(row=2,column=2,rowspan=6) self.list1.configure(yscrollcommand=sb1.set) sb1.configure(command=self.list1.yview) self.list1.bind('<<ListboxSelect>>',self.get_selected_row) b1=Button(window,text="View all", width=12,command=self.view_command) b1.grid(row=2,column=3) b2=Button(window,text="Search entry", width=12,command=self.search_command) b2.grid(row=3,column=3) b3=Button(window,text="Add entry", width=12,command=self.add_command) b3.grid(row=4,column=3) b4=Button(window,text="Update selected", width=12,command=self.update_command) b4.grid(row=5,column=3) b5=Button(window,text="Delete selected", width=12,command=self.delete_command) b5.grid(row=6,column=3) b6=Button(window,text="Close", width=12,command=window.destroy) b6.grid(row=7,column=3) def get_selected_row(self,event): index=self.list1.curselection()[0] self.selected_tuple=self.list1.get(index) self.e1.delete(0,END) self.e1.insert(END,self.selected_tuple[1]) self.e2.delete(0,END) self.e2.insert(END,self.selected_tuple[2]) self.e3.delete(0,END) self.e3.insert(END,self.selected_tuple[3]) self.e4.delete(0,END) self.e4.insert(END,self.selected_tuple[4]) def view_command(self): self.list1.delete(0,END) for row in database.view(): self.list1.insert(END,row) def search_command(self): self.list1.delete(0,END) for row in database.search(self.title_text.get(),self.author_text.get(),self.year_text.get(),self.isbn_text.get()): self.list1.insert(END,row) def add_command(self): database.insert(self.title_text.get(),self.author_text.get(),self.year_text.get(),self.isbn_text.get()) self.list1.delete(0,END) self.list1.insert(END,(self.title_text.get(),self.author_text.get(),self.year_text.get(),self.isbn_text.get())) def delete_command(self): database.delete(self.selected_tuple[0]) def update_command(self): database.update(self.selected_tuple[0],self.title_text.get(),self.author_text.get(),self.year_text.get(),self.isbn_text.get()) window=Tk() Window(window) window.mainloop() And below you will also find the backend.py script in OOP: #backend.py import sqlite3 class Database: def __init__(self, db): self.conn=sqlite3.connect(db) self.cur=self.conn.cursor() self.cur.execute("CREATE TABLE IF NOT EXISTS book (id INTEGER PRIMARY KEY, title text, author text, year integer, isbn integer)") self.conn.commit() def insert(self,title,author,year,isbn): self.cur.execute("INSERT INTO book VALUES (NULL,?,?,?,?)",(title,author,year,isbn)) self.conn.commit() def view(self): self.cur.execute("SELECT * FROM book") rows=self.cur.fetchall() return rows def search(self,title="",author="",year="",isbn=""): self.cur.execute("SELECT * FROM book WHERE title=? OR author=? OR year=? OR isbn=?", (title,author,year,isbn)) rows=self.cur.fetchall() return rows def delete(self,id): self.cur.execute("DELETE FROM book WHERE id=?",(id,)) self.conn.commit() def update(self,id,title,author,year,isbn): self.cur.execute("UPDATE book SET title=?, author=?, year=?, isbn=? WHERE id=?",(title,author,year,isbn,id)) self.conn.commit() def __del__(self): self.conn.close() |  |  |  |